INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC 1/SC 29/WG 11

MPEG2018/M43581

July 2018, Ljubljana, SI

| Source: Status: | Universidad Politécnica de Madrid and Huawei Technologies Co., Ltd. Informative |
| --- | --- |
| Title: Authors: | Results of FTV-Wei project: Real-time free-viewpoint videoconferencing system Pablo Carballeira <pcl@gti.ssr.upm.es>, Daniel Berjón, Sergio Arnaldo, Carlos Carmona, Daniel Corregidor, Carmen Doblado, M. Mar Martín, Julián Cabrera, César Díaz, Francisco Morán, Narciso García (Universidad Politécnica de Madrid) Huawei Technologies Co., Ltd. |

# Introduction

This contribution summarizes the results and main characteristics of the end-to-end free-viewpoint videoconferencing system developed within the FTV-Wei project. Our system works in real time, using off-the-shelf hardware, and texture and depth data captured by consumer-grade cameras. To achieve high visual quality with those restrictions, a novel view synthesis algorithm has been developed, including texture+depth processing tools to enhance the view synthesis quality. Coding and transmission are based on MPEG standards and well-established protocols.

In addition to this document, demo videos showing synthesis results and the real-time operation of the system will be presented at the meeting of the MPEG-I Visual AhG.

# Overview of the FTV-Wei system

Figure 1: FTV-Wei system block scheme

Figure 1 shows the block scheme of the FTV-Wei system, that covers the end-to-end processing chain of (one direction of) a free-viewpoint videoconference. The scheme covers the whole process, from capture to the rendering of the virtual viewpoint, that is displayed in a mobile client, namely a Huawei Mate 9 smartphone. The user can control the virtual viewpoint of the scene, in real time, by swiping his/her finger on the smartphone display: see Figure 2.

Figure 2: Three frames from a video capturing the virtual view displayed on the smartphone in front of the original scene, to show minimum end-to-end delay and the real-time user control of the virtual viewpoint by swiping a finger on the smartphone display. Top row: initial viewpoint located at approximately the center of the virtual view range; middle and bottom rows: viewpoint moved towards the left and right respectively

Table 1 shows an overview of the main technical characteristics and the hardware used in the current setting of the FTV-Wei system. This technical overview shows that the system can operate in real-time using off-the-shelf hardware.

Table 1 Overview of the capabilities and hardware of the FTV-Wei system

| Technical characteristics |  |
| --- | --- |
| Resolution: | Full HD (1920x1080p) |
| Frame rate: | 30 fps |
| Total nr. of cameras: | 9 |
| Motion-to-photon delay: | ~170 ms |
| End-to-end delay: | < 400 ms |
| Equipment |  |
| Capture server PCs (x3): | Intel Core i7-6850K @ 3.6 GHz + 64 GB RAM + NVIDIA GEFORCE GTX 1080 + NVIDIA QUADRO P4000 + Stereolab ZED cameras (3x) |
| Synthesis server PC: | Same as above, except for the cameras |
| Client smartphone: | Huawei Mate 9 |

## Capture and calibration

Figure 3: Camera setting for the FTV-Wei project and captured scene

Figure 3 shows the current setting of cameras and the scene captured at GTI-UPM facilities, where a room is currently exclusively devoted to the development and demonstration of the FTV-Wei system. The current multi-camera setting comprises 9 cameras, which can be re-configured and re-calibrated using standard multi-view calibration procedures based on checkerboard patterns (Figure 4).

Figure 4: Multi-camera calibration using checkerboard patterns

## Coding and transmission

The compression of texture and geometry (depth) data is performed using standard MPEG compression schemes. The transmission of the compressed RGB+depth data from the capture servers to the synthesis server is handled by standard transmission protocols.

## View synthesis

The view synthesis algorithm has been designed and implemented to work at full HD in real time (1920x1080p@30fps), while providing high-quality synthesis results from RGB+depth data yielded by consumer-grade cameras. To fulfill these two requirements, the view synthesis algorithm follows a novel scheme and includes several RGB+depth processing tools that cannot be disclosed yet.

Examples of the view synthesis results will be demonstrated at the presentation of the contribution at the MPEG-I Visual AhG, to prove that the view synthesis module of the FTV-Wei project outperforms MPEG’s View Synthesis Reference Software (VSRS) [1] in terms of subjective quality.

# Conclusions

This contribution informs about the finalization of the FTV-Wei project, developed in collaboration between UPM and Huawei. This project demonstrates the interest of industrial partners in free-viewpoint video systems (windowed-6DoF) and encourages MPEG to continue related standardization activities.

# References

ISO/IEC JTC 1/SC 29/WG 11 output doc. N17548, “Exploration Experiments for MPEG-I: Windowed-6DoF”, 122nd MPEG meeting, San Diego, US, April 2018.